# YUKON GEOTHERMAL DATABASE METADATA

Background

The geothermal compilation contains data sets related to geothermal potential in the Yukon. The data included in this dataset are derived from a variety of sources including historical subsurface thermal data compilations. The compilation includes: 1) boreholes from which temperature data was collected in Yukon and near the border in the Northwest Territories; 2) thermal springs, including water chemistry and temperature, in Yukon and in bordering areas of British Columbia, Northwest Territories and Alaska; and 3) radiogenic heat production values calculated from lithogeochemical analyses compiled by the Yukon Geological Survey (https://data.geology.gov.yk.ca/Compilation/35). These data are presented as overlay to the Curie point depth map of Yukon, calculated using global data of Li et al. (2017).

Scope

The intended audience for this document is the members of the public interested in geothermal potential in the Yukon.

Data Overview

Below are the published geothermal datasets:

- Boreholes

- Thermal springs

- Radiogenic heat production

- Curie Point Depth

# Layer attributes

| BOREHOLES |  |
| --- | --- |
| The borehole layer represents the compilation of available data from various boreholes throughout the Yukon and near the Yukon border. This data set includes oil and gas exploration wells, mineral exploration boreholes, water supply wells and environmental monitoring wells. |  |
| NAME | Common or published name or identifier of the feature. Some features have different names in various publications. |
| LATITUDE_DD | Feature location latitude in GCS_WGS_1984 |
| LONGITUDE_DD | Feature location longitude in GCS_WGS_1984 |
| ELEVATION | Ground elevation at feature in metres above mean sea level (m asl) |
| LOC_SOURCE | Feature locations are derived from a variety of sources including published coordinates, published location descriptions. Where possible, locations have been verified using digital imagery from Google Earth and/or ArcMap Earth applications. |
| LOCATION | Refers to the community, mine, or other major infrastructure reference nearest to the feature |
| PURPOSE | Purpose of the borehole or well – i.e., oil exploration, mineral exploration, water supply, environmental monitoring etc. |
| STATUS | Status of boreholes and wells (i.e., in use, decommissioned, abandoned) are recorded where known. |
| SCREEN_INT | Screened or open interval for water wells in metres below ground surface (m bgs) |
| STATIC_WL | Static water level in metres below ground surface (m bgs) |
| CAPACITY | Well rated production capacity in litres/second (L/s) |
| DISCHARGE | Artesian flowing well discharge rate in litres/second (L/s) |
| TEMP_WATER | Water temperature from a production well measured in degrees Celsius |
| DRILL_DEPTH | Maximum drilled depth of the borehole in metres below ground surface (m bgs) is the depth to which a borehole was advanced. |
| BROCK_DEPTH | Depth to the top of bedrock in metres below the ground surface (m bgs) |
| TEMP_MIN | Minimum measure temperature of a feature in degrees Celsius. Note: this does not necessarily correspond to the measured thermal gradient interval, as spikes and lows in temperature are not necessarily included in a thermal gradient calculation. |
| TEMP_MAX | Maximum measured temperature of a feature in degrees Celsius. Note: this does not necessarily correspond to the measured thermal gradient interval, as spikes and lows in temperature are not necessarily included in a thermal gradient calculation. |
| INT_TOP | Top of the measured thermal gradient interval in metres below ground surface (m bgs) |
| INT_BOTTOM | Bottom of the measured thermal gradient interval in metres below ground surface (m bgs) |
| INT_THERM | Interval over which the thermal gradient is calculated in metres |
| BOTTOM_UNK | Drilled depth or well bottom is unknown |
| THERM_GRAD | Thermal gradient measured in degrees Celsius per kilometre below ground surface (°C/km) |
| TG_CONFIDENCE | TG_Confidence is a rating of data quality confidence based on the number of temperature measurements collected and whether the bore was allowed to thermally equilibrate between drilling and data collection. Further details on the confidence rating method are discussed below. |
| TG_METHOD | Method of thermal measurement collection: BHT = Bottom Hole Temperature is collected at the bottom of a borehole after drilling. The logs in this data set are collected both directly after drilling and over the course of several months or years. SLT = Single Log Temperature is collected in a single log run and can be collected directly after drilling and over the course of several months and years. Single thermistor probe is lowered down the hole and allowed to equilibrate at each measured depth to collect a single measurement from that interval. DST = Drill Stem Tests are conducted while drilling. Because these tests are done during drilling, temperature measurements will not be accurate as the well is not allowed to equilibrate. A multi thermistor probe has several nodes and is lowered down the hole, allowed to equilibrate to collect temperature measurements from multiple depths. Multi-thermistor logs can be collected directly after drilling or after well equilibration as long as the hole is open. Continuous = temperature profile collected: 1) using a high resolution probe taking continuous temperature and pressure readings as it is lowered in the borehole; or 2) measured using a Distributed Temperature Sensor (DTS) fiber optic cable installed in the borehole. |
| TG_CORRECTION | Statistical correction applied to thermal gradient for paleoclimate or topographic factors. |
| TG_DATE | Date refers to the date(s) on which temperature measurements or logs were collected. |
| TG_LOG_NUM | Log numbers refers to the log identifier number for a given thermal gradient measurement in the reference file. |
| THERM_COND | Thermal conductivity is a measure of the thermal conductivity of the subsurface material (soil or rock) in a borehole expressed in milliwatts per metre-Kelvin (mW/mK) |
| HEAT_FLOW | Heat flow is a measure of how much energy is transferred through a given material (soil or rock) measured in milliwatts per metre squared (mW/m2) |
| HF_CORRECTION | Statistical correction applied to heat flow calculation for paleoclimate or borehole location topography |
| GT_SILICA | Silica geothermometer results from measured silica or silicon concentration in water expressed in degrees Celsius. Silica geothermometer calculations used the following formula based on the updated Fournier method developed by Verma and Santoyo (1995). This method is applicable for temperatures ranging from 20°C to 210°C. T°C= -44.119+0.24469SiO2-1.7414 E-4SiO2+79.305log⁡(SiO2) |
| GT_ALKALI | Alkalinity geothermometer results from measured sodium, potassium and calcium concentration expressed in degrees Celsius. Alkaline geothermometer calculations were completed using the Fournier and Truesdell (1973) formula because calcium is the dominant cation in the water chemistry where the alkali geothermometer was calculated. The formula used is below: T°C={1647logNaK+BlogCaNa+2.06+2.47-273.15 |
| PH | Measured potential of hydrogen in water either from laboratory testing or field measurement |
| CONDUCTIVITY | Electrical conductivity measured in water expressed in micro-Siemens per centimetre (µS/cm) |
| HARDNESS | Water hardness expressed in milligrams per litre (mg/L) |
| CA_MG_L | Calcium concentration measured in water expressed in milligrams per litre. |
| MG_MG_L | Magnesium concentration measured in water expressed in milligrams per litre. |
| NA_MG_L | Sodium concentration measured in water expressed in milligrams per litre. |
| K_MG_L | Potassium concentration measured in water expressed in milligrams per litre. |
| HCO3_MG_L | Bicarbonate concentration measured in water expressed in milligrams per litre. |
| SO4_MG_L | Sulphate concentration measured in water expressed in milligrams per litre |
| CO3_MG_L | Carbonate concentration measured in water expressed in milligrams per litre. |
| SIO2_MG_L | Chloride concentration measured in water expressed in milligrams per litre. |
| CL_MG_L | Fluoride concentration measured in water expressed in milligrams per litre. |
| F_MG_L | Silica concentration measured either directly measured or calculated from dissolved or total silicon measured in water expressed in milligrams per litre. |
| E_ISO_18O | Environmental isotopes measured in water expressed in parts per thousand for Oxygen-18 |
| E_ISO_2H | Environmental isotopes measured in water expressed in parts per thousand for Deuterium (Hydrogen-2) |
| RAD_ISO_3H | Radio isotopes measured in water expressed as concentration of tritium (Hydrogen-3) in tritium units |
| GAS_NOBLE | Noble gasses detected in water – refer to reference for values. |
| GAS_DISS | Dissolved gasses expressed in mg/K for carbon dioxide and µg/L for hydrogen sulphide |
| GAS_EMITTED | Gasses emitted or off gassed from thermal water expressed in Mole %. |
| REFERENCE | Reference the data was compiled from. |
| REF_LINK | Link to relevant reference material provided where available. |
| COMMENTS | Comments on feature characteristics, background data variability and other observations |
| PUBLIC | Permission given to share publicly (Yes/No) |

| THERMAL SPRINGS |  |
| --- | --- |
| The thermal springs layer represents a compilation of available data from thermal springs throughout the Yukon and near the Yukon border. |  |
| NAME | Common or published name or identifier of the thermal water feature. Some features have different names in various publications. Spring names are followed by an abbreviated designation as either a hot or warm spring. The threshold for warm vs. hot is defined in this compilation as 30°C. - HS = Hot spring (30 °C or greater water temperature) - WS = Warm Spring (10°C to 30°C water temperature) |
| LATITUDE_DD | Feature location latitude in GCS_WGS_1984 |
| LONGITUDE_DD | Feature location longitude in GCS_WGS_1984 |
| LOC_SOURCE | Feature locations are derived from a variety of sources including published coordinates, published location descriptions. Where possible, locations have been verified using digital imagery from Google Earth and/or ArcMap Earth applications. |
| WATER_TEMP | Spring water temperature measured in degrees Celsius. Highest measured temperatures are presented here, additional data are included in some reference reports. |
| DISCHARGE | Spring discharge rate measured or estimated in litres/second (L/s) |
| GT_SILICA | Silica geothermometer results from measured silica or silicon concentration in water expressed in degrees Celsius. Silica geothermometer calculations used the following formula based on the updated Fournier method developed by Verma and Santoyo (1995). This method is applicable for temperatures ranging from 20°C to 210°C. T°C= -44.119+0.24469SiO2-1.7414 E-4SiO2+79.305log⁡(SiO2) |
| GT_ALKALI | Alkalinity geothermometer results from measured sodium, potassium and calcium concentration expressed in degrees Celsius. Alkaline geothermometer calculations were completed using the Fournier and Truesdell (1973) formula because calcium is the dominant cation in the water chemistry where the alkali geothermometer was calculated. The formula used is below: T°C={1647logNaK+BlogCaNa+2.06+2.47-273.15 |
| PH | Measured potential of hydrogen in water either from laboratory testing or field measurement |
| CONDUCTIVITY | Electrical conductivity measured in water expressed in micro-Siemens per centimetre (µS/cm) |
| HARDNESS | Water hardness expressed in milligrams per litre (mg/L) |
| CA_MG_L | Calcium concentration measured in water expressed in milligrams per litre. |
| MG_MG_L | Magnesium concentration measured in water expressed in milligrams per litre. |
| NA_MG_L | Sodium concentration measured in water expressed in milligrams per litre. |
| K_MG_L | Potassium concentration measured in water expressed in milligrams per litre. |
| HCO3_MG_L | Bicarbonate concentration measured in water expressed in milligrams per litre. |
| SO4_MG_L | Sulphate concentration measured in water expressed in milligrams per litre |
| CO3_MG_L | Carbonate concentration measured in water expressed in milligrams per litre. |
| CL_MG_L | Chloride concentration measured in water expressed in milligrams per litre. |
| F_MG_L | Fluoride concentration measured in water expressed in milligrams per litre. |
| SIO2_MG_L | Silica concentration measured either directly measured or calculated from dissolved or total silicon measured in water expressed in milligrams per litre. |
| E_ISO_18O | Environmental isotopes measured in water expressed in parts per thousand for Oxygen-18. |
| E_ISO_2H | Environmental isotopes measured in water expressed in parts per thousand for Deuterium (Hydrogen-2). |
| RAD_ISO_3H | Radio isotopes measured in water expressed as concentration of tritium (Hydrogen-3) in tritium units. |
| RAD_ISO_14C | Radiocarbon (Carbon-14) expressed in percent modern carbon. |
| GAS_NOBLE | Noble gasses detected in water – refer to reference for values. |
| GAS_DISS | Dissolved gasses expressed in mg/K for carbon dioxide and µg/L for hydrogen sulphide. |
| GAS_EMITTED | Gasses emitted or off gassed from thermal water expressed in Mole %. |
| REFERENCE | Reference the data was compiled from. |
| REF_LINK | Link to relevant reference material provided where available. |
| COMMENTS | Comments on feature characteristics, background data variability and other observations. |
| PUBLIC | Permission given to share publicly (Yes/No). |

| Radiogenic Heat Production |  |
| --- | --- |
| This dataset presents results for calculations of the potential for granitoid plutons to produce radiogenic heat. The calculations are performed using U, Th, and K concentrations from whole rock geochemical data compiled from various sources in the Yukon Geological Survey lithogeochemical compilation (https://data.geology.gov.yk.ca/Compilation/35). Plutons with anomalously high potential for radiogenic heat production (>2.8 µW/m3) may help target exploration for geothermal resources. |  |
| SAMPLE_NUM | Sample Number |
| AGE | Age of pluton/sample |
| P_SUITE | Plutonic Suite |
| BATH_PLUT | Name of the Batholith or Pluton the sample is derived from |
| DESC | Description of sample |
| LATITUDE_DD | Feature location latitude in GCS_WGS_1984 |
| LONGITUDE_DD | Feature location longitude in GCS_WGS_1984 |
| LOC_ACC | Presumed accuracy of coordinates |
| COMP | Composition of sample (e.g., felsic, intermediate, mafic) |
| ᴩ_kg_m3 | Density of sample (in kilogram per cubic meter) |
| K2O | Potassium Oxide |
| cTh | Concentration of Thorium |
| cU | Concentration of Uranium |
| cK | Concentration of Potassium |
| A_µW_m3 | Potential radiogenic heat production (micro-Watt per cubic meter) |

# Curie Point Depth mapping

The Curie point is the temperature at which certain materials lose their permanent magnetic properties. For magnetite, the most common magnetic mineral, the Curie temperature is 580°C. Therefore, the Curie point depth (CPD) is the depth at which a temperature of ~580°C is encountered in the Earth’s crust. The calculation of Curie point is done using regional aeromagnetic survey data (e.g., Witter and Miller, 2017; Witter et al., 2018).

The Curie point depth map for Yukon was generated using data from Li et al. (2017). This map shows the depth to the 580°C isotherm and can be used as a proxy for estimation of the geothermal gradient. Shallower CPD may be indicative of higher heat flow and geothermal gradient, and could be used as a regional targeting tool for geothermal exploration.

# Thermal Data Confidence Rating

Thermal gradient data confidence rating is based on the number of data points in a given thermal gradient record, and the period between drilling completion and thermal data collection where reported.

The rating is based on the following table. The higher the confidence the higher the rating number with a 15 representing a data set where the gradient is based on continuous or near continuous data with a clear thermal recovery period of one or more months, and a rating of 1 representing a thermal gradient based on a single downhole temperature measurement extrapolated to surface with no clear thermal equilibration period reported. Boreholes where no thermal gradient has been recorded have been assigned a thermal gradient confidence rating of zero (0).

Drill stem tests (DST) which involve formation fluid extraction in a given section and Horner corrected bottom hole temperature (BHT) measurements from existing compilations are considered to sample thermally recovered formation temperature with moderate confidence.

| Increasing number of data points | ≥ 5 | 5 | 10 | 15 |
| --- | --- | --- | --- | --- |
|  | 4 | 4 | 8 | 12 |
|  | 3 | 3 | 6 | 9 |
|  | 2 | 2 | 4 | 6 |
|  | 1 | 1 | 2 | 3 |
|  | 0 | 1 | 2 | 3 |
|  |  | No thermal recovery period | DST or Horner corrected BHT | Thermally recovered |

0 – No thermal data exists.

1 – Thermal gradient based on a single bottom-hole temperature extrapolated to surface; measured directly after drilling or no thermal equilibration period reported.

2 – Thermal gradient based on a single downhole measurement with a thermal equilibration correction or thermal equilibration period of less than one month; or gradient based on 2 to 5 measurements with no thermal equilibration period.

3 – Thermal gradient based on 6 to 10 measurements with no thermal equilibration period reported; or thermal gradient based on one data point with high confidence thermal equilibration period.

4 – Thermal gradient based on 2 to 5 measurements with a thermal equilibration correction or thermal equilibration less than one month; or thermal gradient based 11 to 15 temperature measurements with no reported thermal equilibration period.

5 – Thermal gradient based on more than 15 measurements with no thermal equilibration period.

6 – Thermal gradient based on 6 to 10 temperature measurements with a thermal equilibration correction; or thermal gradient based on 2 to 5 measurements with a thermal equilibration period.

8 – Thermal gradient based on 11 to 15 temperature measurement with a thermal equilibration correction.

9 – Thermal gradient based on 6 to 10 data points with a thermal equilibration period.

10 – Thermal gradient based on more than 15 temperature measurements with a thermal equilibration correction.

12 – Thermal gradient based on 11 to 15 temperature measurements with a thermal equilibration period.

15 – Thermal gradient based on more than 15 data points with a thermal equilibration period.

# References:

Fournier, R. O. and Truesdell, A. H., 1973. An empirical Na-K-Ca geothermometer for natural waters. Geochimica et Cosmochimica Acta, vol. 37, p. 1255-1275.

Li, C.-F., Lu, Y. and Wang, J., 2017. A global reference model of Curie-point depths based on EMAG2. Scientific Reports, vol. 7, p. 45129, DOI: 10.1038/srep45129.

Verma, S. P. and Santoyo, E., 1997. New improved equations for Na/K, Na/Li and SiO2 geothermometers by outlier detection and rejection. Journal of Volcanology and Geothermal Research, vol. 79, p. 9-23.

Witter, J. and Miller, C., 2017. Curie point depth mapping in Yukon. Yukon Geological Survey, Open File 2017-3, 37 p.

Witter, J.B., Miller, C.A., Friend, M. and Colpron, M., 2018. Curie point depths and heat production in Yukon, Canada.  43rd Workshop on Geothermal Reservoir Engineering: Stanford University, CA, 11 p.